컴퓨터구조 기말시험 답안지(2021/06/07)

[1] 10점

| step | 1-bus |  |
| --- | --- | --- |
|  | concrete RTN | control sequence |
| T3 | R[ra]<-PC; | PCout, Gra, Rin |
| T4 | CON<-cond(R[rc]); | Grc, Rout, CONin |
| T5 | CON->PC<-R[rb]; | Grb, Rout, CON->PCin,End |

[2] 15점

| Control store 주소 | Mux control |  | 분기조건 및 제어신호들 | Branch address | Actions |
| --- | --- | --- | --- | --- | --- |
| 100 | 0 | 0 | 여기는 채울 필요없음 | XXX | MA ← PC: C ← PC+4 |
| 101 | 0 | 0 |  | XXX | MD <- M[MA]: PC <- C; |
| 102 | 0 | 1 |  | XXX | IR <- MD: uPC <- PLA; |
| 200 | 0 | 0 |  | XXX | A ← R[rb]; |
| 201 | 0 | 0 |  | XXX | C ← A - R[rc]; |
| 202 | 1 | 1 |  | 100 | R[ra] <- C: uPC <- 100; |
| 300 | 0 | 0 |  | XXX | A ← R[rb]; |
| 301 | 0 | 0 |  | XXX | C <- A + (16@IR<16>#IR<15..0>); |
| 302 | 1 | 1 |  | 100 | R[ra] <- C: uPC <- 100; |

[3] 15점

| step | 1-bus |  |
| --- | --- | --- |
|  | RTN | control sequence |
| T0 | MA <- PC: C<- PC + 4; | PCout, MAin, INC4, Cin, Read |
| T1 | MD <- M[MA]: PC <- C; | Cout, PCin, Wait |
| T2 | IR <- MD; | MDout, IRin |
| T3 | A <- R[rb]; | Grb, Rout, Ain |
| T4 | C <- A + (16@IR<16>#IR<15..0>); | c2out, ADD, Cin |
| T5 | R[ra] <- C; | Cout, Gra, Rin, End |

| step | 2-bus |  |
| --- | --- | --- |
|  | RTN | control sequence |
| T0 | MA <- PC; | PCout, C=B, MAin, Read |
| T1 | PC <- PC + 4: MD<-M[MA]; | PCout, INC4, PCin, Wait |
| T2 | IR <- MD; | MDout, C=B, IRin |
| T3 | A <- R[rb]; | Grb, Rout, C=B, Ain |
| T4 | R[ra] <- A + (16@IR<16>#IR<15..0>); | c2out, ADD, Sra, Rin, End |
| T5 |  |  |

| step | 3-bus |  |
| --- | --- | --- |
|  | RTN | control sequence |
| T0 | MA <- PC: PC <- PC+4: MD <- M[MA]; | PCout, MAin, INC4, PCin, Read, Wait |
| T1 | IR <- MD; | MDout, C=B, IRin |
| T2 | R[ra] <- R[rb] + (16@IR<16>#IR<15..0>); | c2Bout, GArb, RAout, ADD, Sra, Rin, End |
| T3 |  |  |
| T4 |  |  |
| T5 |  |  |